[image: image1.png]crmy or /AN

LAKE (HLSINORE





What is Mobile Pressure Washing?
Pressure washing is defined as “power washing, steam cleaning, and any other mobile cosmetic cleaning operation (with or without detergents or chemical additives) of various surfaces including, but not limited to, transportation-related, structural and food-related cleaning engaged in for commercial purposes.”
What are the potential hazards from mobile pressure washing?

Wash water from vehicle and equipment operations contains a variety of pollutants which can harm aquatic life and the environment and contaminate recreational sites and drinking water supplies, even if cleaning solutions are labeled nontoxic or biodegradable.

Vehicle/equipment washing/steam cleaning has the potential to contribute metals, oil and grease, detergents, degreasing chemicals, solvents, phosphates, organics, acids, caustics and suspended solids to the storm water drainage system and receiving waters.
Disposing of these materials into storm drains, ditches, and gutters is prohibited by law.

Regulations Prohibiting Non-Storm Water Surface Discharge:

· Lake Elsinore Municipal Code Section 14.080.070

· State of California Porter-Cologne Water Quality Control Act, Division 7, Chapter 5.5, Section 13376

· Federal Clean Water Act, 40 CFR Parts 122-124

· Santa Ana Region NPDES Storm Water Permit
What are Best Management Practices (BMP’s)?

Best Management Practices (BMP’s) are procedures that help prevent pollutants from entering our storm drains.  
BMP requirements for mobile vehicle/equipment washing include:

Minimize Pollutants Generated.

· Use a mop or rags to pre-clean the vehicle or equipment.

· Minimize water use by using high pressure, low volume nozzles.

· Use the least amount and toxic detergents and degreasers  (Phosphate Free or Biodegradable) to complete the job.

[image: image2.png]crmy or /AN

LAKE (HLSINORE






Contain and Collect Wash Water.

· All wash water containment and removal equipment shall be installed and operational prior to commencement of any cleaning processes or wastewater generation activities.

· Common equipment used for containing and collecting wastewater generated during pressure washing activities includes: vacuum pumps, booms/berms, portable containment areas/mats, weighted storm drain covers, inflatable plumber’s plugs, oil/water separators, holding tanks, portable sump pumps, hoses, and absorbents.

· Storm drain inlets located within or down grade of wash areas shall be covered or otherwise protected to prevent entry of wash water.

· Vacuum booms shall be used at all times and arranged in such a fashion to direct all wastewater generated in the vacuum ports in the vacuum boom.
· All wastewater recovery equipment shall be used in accordance with manufacturer’s recommendations.

Properly Dispose of Wash Water.

· All wastewater generated from cleaning activities shall be collected and treated for reuse or drained to the sanitary sewer through an interior building drain, sink or private sewer clean-out with the owner’s permission.
· All discharges to the sanitary sewer must be in accordance with local sanitary sewer disposal regulations.

· No hazardous waste, such as engine degreasers or toxic wheel cleaners, shall be discharged to the sanitary sewer system.

· Contact Elsinore Valley Municipal Water District (951) 674-3146 for proper sewer disposal methods.
Only Rain Down the Storm Drain.

· At no time shall any wastewater be discharged to the storm drain system.  This includes driveways, streets, and gutters.

Leave No Residue Behind.

· Any wash chemical residue generated shall be rinsed from the surface and removed with the wastewater.

· No wash chemical residue is to remain on the washing surface.

Businesses conducting pressure washing activities within the City must comply with all above BMP’s.  Businesses that do not adequately implement these BMP’s are in violation of Lake Elsinore Municipal Code and are subject to enforcement actions.

�                         �








�  





ONLY RAIN DOWN THE STORM DRAIN!





For More Information please call City of Lake Elsinore’s NPDES Coordinator at 951-674-3124, extension 308 for questions regarding proper implementation of the BMP’s.














